BWTS CHECK LIST

| Maker : PANASIA | Model : GloEn-Patrol | Type : Filter + UV |
| --- | --- | --- |
| Hull No. : | Vessel Name : | IMO No. : |

1. Safety

[General instruction] - Prior to installation and/or operation, review the installation and operation instructions thoroughly - While operating the equipment, all conventional safety instructions should be observed in order to avoid harm to operator property and others in the vicinity. - No alterations or modifications to the equipment are permitted without written approval given by the manufacturer or representative on the manufacturer’s behalf. [Operation, control and maintenance] - If you need to disassemble the equipment, turn off the power from Control panel & UV power supply panel. - Avoid moisture build-up (i.e. splashing and/or water leaks) minimize slipping or electrical dangers. - Use authorized spare parts only for maintenance.

2. Before operation

| No. | Instruction | Result |
| --- | --- | --- |
| 1 | Power supply to the control panel and UV power supply panel. | □ Good □ Poor |
| 2 | Make sure the cable for each equipment and control panel are connected properly. | □ Good □ Poor |
| 3 | Grease on the shaft and bushing at Filter. | □ Good □ Poor |
| 4 | Check each equipment for leakage & damage visually. | □ Good □ Poor |
| 5 | Check there is alarms on monitor. | □ Good □ Poor |
| 6 | Ship’s control valves open and close operating & signal normally. | □ Good □ Poor |

3. During operation

| Equipment | Check point | Result |
| --- | --- | --- |
| Filter unit | Filter inlet pressure value is higher 0.5kg/cm2 | □ Yes □ No |
|  | Filter DP pressure value is less 0.45kg/cm2 | □ Yes □ No |
|  | When DP value is higher 0.45kg/cm2, Back flushing motor operate normally. | □ Yes □ No |
|  | When DP value is higher 0.45kg/cm2, Back flushing valve open normally. | □ Yes □ No |
|  | No Filter Pressure sensor fail alarm. | □ Yes □ No |
| UV unit UV unit | Dose value higher than 300 mJ/㎠ or UVI value higher than 73 mW/㎠ | □ Yes □ No |
|  | UV wiper working normally when Dose value below 300 mJ/㎠ or UVI value is below 73 mW/㎠ | □ Yes □ No |
|  | UV unit temperature below 50 ℃ | □ Yes □ No |
|  | No UV intensity sensor fail alarm. | □ Yes □ No |
|  | No UV temperature sensor fail alarm. | □ Yes □ No |
|  | Sea to sea operation during cooling time after UV stop | □ Yes □ No |
| Control panel | Control panel FAN is operating normally. | □ Yes □ No |
|  | Temperature inside control panel is below 50 ℃ | □ Yes □ No |
|  | HMI screen touch working normally. | □ Yes □ No |
| ETC | EHS valves open close normally. | □ Yes □ No |
|  | Other valves signal is display normally. | □ Yes □ No |
|  | When pump is starting, pump running signal is received. | □ Yes □ No |
|  | Flow rate is normal value. | □ Yes □ No |
|  | No alarm from other sensor | □ Yes □ No |
| MODE OPERATION | Ballast mode | □ Normal □Abnormal |
|  | De-ballast mode | □ Normal □Abnormal |
|  | Eductor mode (If don’t have Eductor mode, ignore it) | □ Normal □Abnormal |
|  | *Remark: 1) Please explain the situations in case of abnormal conditions during each operating condition. 2) If have any records of alarms or trips, please write them in this sheet and attach the photos. |  |

4. After operation

| No. | Instruction | Result |
| --- | --- | --- |
| 1 | EHS valves close normally. | □ Good □ Poor |
| 2 | Any leakage & damage on each equipment visually. | □ Good □ Poor |
| 3 | Cable terminal burn damage inside UV power supply panel. | □ Good □ Poor |
| 4 | Log extraction via USB is working normally. | □ Good □ Poor |

Tested by